Lab Report
Lab 11: Interrupts & IO programming

| Student ID | 20235890 |
| --- | --- |
| Student Name | Nguyễn Đức Anh |

Assignment 1:

Code:

# ------------------------------------------------------

#           col 0x1     col 0x2     col 0x4     col 0x8

# row 0x1       0           1           2           3

#             0x11        0x21        0x41        0x81

# row 0x2       4           5           6           7

#             0x12        0x22        0x42        0x82

# row 0x4       8           9           a           b

#             0x14        0x24        0x44        0x84

# row 0x8       c           d           e           f

#             0x18        0x28        0x48        0x88

# ------------------------------------------------------

# Command row number of hexadecimal keyboard (bit 0 to 3)

# Eg. assign 0x1, to get key button 0,1,2,3

# assign 0x2, to get key button 4,5,6,7

# NOTE must reassign value for this address before reading,

# eventhough you only want to scan 1 row

.eqv IN_ADDRESS_HEXA_KEYBOARD 0xFFFF0012

# Receive row and column of the key pressed, 0 if not key pressed

# Eg. equal 0x11, means that key button 0 pressed.# Eg. equal 0x28, means that key button D pressed.

.eqv OUT_ADDRESS_HEXA_KEYBOARD 0xFFFF0014

.data

space: .asciz " "

.text

main:

li t1, IN_ADDRESS_HEXA_KEYBOARD

li t2, OUT_ADDRESS_HEXA_KEYBOARD

li t4, 0x08 # declare the maximum row

init_row_1:

li t3, 0x01 # init the row 1

polling:

sb t3, 0(t1) # must reassign expected row

lb a0, 0(t2) # read scan code of key button

bne a0, zero, print # if a0 = 0, jump to print

next_row:

slli t3, t3, 1 # shift left 1 bit to get next row

check_row:

bgt t3, t4, init_row_1 # check if the current row is greater than maximum row -> jump back to row 1

print:

li a7, 34 # print integer (hexa)

ecall

print_space:

la a0, space # print thye space

li a7, 4

ecall

sleep:

li a0, 100 # sleep 100ms

li a7, 32

ecall

back_to_polling:

j polling # continue polling

Explaination: The Algorithms is we shift left 1 bit to take the next value means next row and using branches condition to check the row not greater than 4 means 0x08.

Result:

Assignment 2:

Code:

.eqv IN_ADDRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADDRESS_HEXA_KEYBOARD 0xFFFF0014

.data

message: .asciz "Someone's presed a button.\n"

.text

main:

# Load the interrupt service routine address to the UTVEC register

la t0, handler

csrrs zero, utvec, t0

# Set the UEIE (User External Interrupt Enable) bit in UIE register

li t1, 0x100

csrrs zero, uie, t1 # uie - ueie bit (bit 8)

# Set the UIE (User Interrupt Enable) bit in USTATUS register

csrrsi zero, ustatus, 0x1 # ustatus - enable uie (bit 0)

# Enable the interrupt of keypad of Digital Lab Sim

li t1, IN_ADDRESS_HEXA_KEYBOARD

li t3, 0x80 # bit 7 = 1 to enable interrupt

sb t3, 0(t1)

# --------------------------------------------------------------

# No-end loop, main program, to demo the effective of interrupt

# --------------------------------------------------------------

loop:

nop

# Delay 10ms

li a7, 32

li a0, 10

ecall

nop

j loop

end_main:

# -----------------------------------------------------------------

# Interrupt service routine

# -----------------------------------------------------------------

handler:

# ebreak # Can pause the execution to observe registers

# Saves the context

addi sp, sp, -8

sw a0, 0(sp)

sw a7, 4(sp)

# Handles the interrupt

# Shows message in Run I/O

li a7, 4

la a0, message

ecall

# Restores the context

lw a7, 4(sp)

lw a0, 0(sp)

addi sp, sp, 8

# Back to the main procedure

uret

Explaination:

.eqv IN_ADDRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADDRESS_HEXA_KEYBOARD 0xFFFF0014

.data

message: .asciz "Someone's presed a button.\n"

- IN_ADDRESS_HEXA_KEYBOARD and OUT_ADDRESS_HEXA_KEYBOARD define specific memory-mapped addresses for input and output related to the keypad interrupt.

- “message” defines a string in memory to be displayed when an interrupt occurs.

- Load the Handler Address:

main:

# Load the interrupt service routine address to the UTVEC register

la t0, handler

csrrs zero, utvec, t0

- la t0, handler loads the address of the interrupt service routine (handler) into t0.

- csrrs zero, utvec, t0 sets this address in the utvec register, which stores the user interrupt vector base address.

- Enable External User Interrupts:

# Set the UEIE (User External Interrupt Enable) bit in UIE register

li t1, 0x100

csrrs zero, uie, t1 # uie - ueie bit (bit 8)

- li t1, 0x100 sets the 8th bit (UEIE) of the uie register, enabling user-level external interrupts.

- Enable Global User Interrupts:

# Set the UIE (User Interrupt Enable) bit in USTATUS register

csrrsi zero, ustatus, 0x1 # ustatus - enable uie (bit 0)

- csrrsi zero, ustatus, 0x1 sets the 0th bit (UIE) in the ustatus register, enabling the global interrupt system.

- Enable Keypad Interrupt:

# Enable the interrupt of keypad of Digital Lab Sim

li t1, IN_ADDRESS_HEXA_KEYBOARD

li t3, 0x80 # bit 7 = 1 to enable interrupt

sb t3, 0(t1)

- The memory address IN_ADDRESS_HEXA_KEYBOARD is loaded into t1.

- sb t3, 0(t1) stores 0x80 (bit 7 set) to enable keypad interrupts.

- Main program loop:

loop:

nop

# Delay 10ms

li a7, 32

li a0, 10

ecall

nop

j loop

end_main:

- This loop keeps the program running indefinitely (nop instructions do nothing).

- li a7, 32 and li a0, 10 prepare for an environment call (ECALL) to delay for 10ms.

- ecall invokes the delay system call.

- Interrupt Service Routine (ISR)

- Save the context:

# ebreak # Can pause the execution to observe registers

# Saves the context

addi sp, sp, -8

sw a0, 0(sp)

sw a7, 4(sp)

Saves a0 and a7 (used registers) onto the stack to preserve the program's state.

- Handle the Interrupt:

# Handles the interrupt

# Shows message in Run I/O

li a7, 4

la a0, message

ecall

li a7, 4 and la a0, message set up to display the string message.

ecall invokes a system call to output the message.

- Restore the Context:

# Restores the context

lw a7, 4(sp)

lw a0, 0(sp)

addi sp, sp, 8

Restores a0 and a7 from the stack and adjusts the stack pointer.

- Return from Interrupt

# Back to the main procedure

Uret

uret resumes execution from the point where the interrupt occurred.

Result:

Assignment 3:

Code:

.eqv IN_ADDRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADDRESS_HEXA_KEYBOARD 0xFFFF0014

.data

message: .asciz "Key scan code: "

# -----------------------------------------------------------------

# MAIN Procedure

# -----------------------------------------------------------------

.text

main:

# Load the interrupt service routine address to the UTVEC register

la t0, handler

csrrw zero, utvec, t0

# Set the UEIE (User External Interrupt Enable) bit in UIE register

li t1, 0x100

csrrs zero, uie, t1  # Enable external interrupts in the user mode

# Set the UIE (User Interrupt Enable) bit in USTATUS register

csrrsi zero, ustatus, 0x1  # Enable interrupts globally

# Enable the interrupt of keypad of Digital Lab Sim

li t1, IN_ADDRESS_HEXA_KEYBOARD

li t3, 0x80  # Enable interrupt (bit 7)

sb t3, 0(t1)

# ---------------------------------------------------------

# Loop to print a sequence of numbers

# ---------------------------------------------------------

xor s0, s0, s0  # count = s0 = 0

loop:

addi s0, s0, 1  # Increment count

prn_seq:

addi a7, zero, 1

add a0, s0, zero  # Print auto sequence number

ecall

addi a7, zero, 11

li a0, '\n'  # Print EOL

ecall

sleep:

addi a7, zero, 32

li a0, 300  # Sleep 300 ms

ecall

j loop  # Jump back to loop

end_main:

# -----------------------------------------------------------------

# Interrupt Service Routine (Handler)

# -----------------------------------------------------------------

handler:

# Save context

addi sp, sp, -16

sw a0, 0(sp)

sw a7, 4(sp)

sw t1, 8(sp)

sw t2, 12(sp)

# Print message

prn_msg:

addi a7, zero, 4

la a0, message

ecall

# Scan all rows to detect pressed key

get_key_code:

li t1, IN_ADDRESS_HEXA_KEYBOARD

li t2, 0x81

sb t2, 0(t1)  # Enable row 1

li t1, OUT_ADDRESS_HEXA_KEYBOARD

lb t3, 0(t1)

bne t3, zero, prn_key_code  # If key pressed, go to print key

li t1, IN_ADDRESS_HEXA_KEYBOARD

li t2, 0x82

sb t2, 0(t1)  # Enable row 2

li t1, OUT_ADDRESS_HEXA_KEYBOARD

lb t3, 0(t1)

bne t3, zero, prn_key_code  # If key pressed, go to print key

li t1, IN_ADDRESS_HEXA_KEYBOARD

li t2, 0x84

sb t2, 0(t1)  # Enable row 3

li t1, OUT_ADDRESS_HEXA_KEYBOARD

lb t3, 0(t1)

bne t3, zero, prn_key_code  # If key pressed, go to print key

li t1, IN_ADDRESS_HEXA_KEYBOARD

li t2, 0x88

sb t2, 0(t1)  # Enable row 4

li t1, OUT_ADDRESS_HEXA_KEYBOARD

lb t3, 0(t1)

bne t3, zero, prn_key_code  # If key pressed, go to print key

j end_handler  # No key press, skip key printing

# Print the key code

prn_key_code:

addi a7, zero, 34  # Print integer syscall

add a0, t3, zero   # Key code to print

ecall

addi a7, zero, 11

li a0, '\n'  # Print EOL

ecall

# Restore context and return

end_handler:

lw t2, 12(sp)

lw t1, 8(sp)

lw a7, 4(sp)

lw a0, 0(sp)

addi sp, sp, 16

uret  # Return from interrupt

Result:

Assignment 4:

Code:

.eqv IN_ADDRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv TIMER_NOW 0xFFFF0018

.eqv TIMER_CMP 0xFFFF0020

.eqv MASK_CAUSE_TIMER 4

.eqv MASK_CAUSE_KEYPAD 8

.data

msg_keypad: .asciz "Someone has pressed a key!\n"

msg_timer: .asciz "Time inteval!\n"

# -----------------------------------------------------------------

# MAIN Procedure

# -----------------------------------------------------------------

.text

main:

# Load the interrupt service routine address to the UTVEC register

la t0, handler

csrrs zero, 5, t0

# Set the UEIE (User External Interrupt Enable) bit in UIE register

li t1, 0x100

csrrs zero, 4, t1 # uie - ueie bit (bit 8) - external interrupt

csrrsi zero, 4, 0x10 # uie - utie bit (bit 4) - timer interrupt

# Set the UIE (User Interrupt Enable) bit in USTATUS register

csrrsi zero, 0, 0x1 # ustatus - enable uie - global interrupt

# ---------------------------------------------------------

# Enable interrupts you expect

# ---------------------------------------------------------

# Enable the interrupt of keypad of Digital Lab Sim

li t1, IN_ADDRESS_HEXA_KEYBOARD

li t2, 0x80 # bit 7 of = 1 to enable interrupt

sb t2, 0(t1)

# Enable the timer interrupt

li t1, TIMER_CMP

li t2, 1000

sw t2, 0(t1)

# ---------------------------------------------------------

# No-end loop, main program, to demo the effective of interrupt

# ---------------------------------------------------------

loop:

nop

li a7, 32

li a0, 10

ecall

nop

j loop

end_main:

# -----------------------------------------------------------------

# Interrupt service routine

# -----------------------------------------------------------------

handler:

# Saves the context

addi sp, sp, -16

sw a0, 0(sp)

sw a1, 4(sp)

sw a2, 8(sp)

sw a7, 12(sp)

# Handles the interrupt

csrr a1, ucause

li a2, 0x7FFFFFFF

and a1, a1, a2 # Clear interrupt bit to get the value

li a2, MASK_CAUSE_TIMER

beq a1, a2, timer_isr

li a2, MASK_CAUSE_KEYPAD

beq a1, a2, keypad_isr

j end_process

timer_isr:

li a7, 4

la a0, msg_timer

ecall

# Set cmp to time + 1000

li a0, TIMER_NOW

lw a1, 0(a0)

addi a1, a1, 1000

li a0, TIMER_CMP

sw a1, 0(a0)

j end_process

keypad_isr:

li a7, 4

la a0, msg_keypad

ecall

j end_process

end_process:

# Restores the context

lw a7, 12(sp)

lw a2, 8(sp)

lw a1, 4(sp)

lw a0, 0(sp)

addi sp, sp, 16

uret

Explaination:

.eqv IN_ADDRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv TIMER_NOW 0xFFFF0018

.eqv TIMER_CMP 0xFFFF0020

.eqv MASK_CAUSE_TIMER 4

.eqv MASK_CAUSE_KEYPAD 8

.data

msg_keypad: .asciz "Someone has pressed a key!\n"

msg_timer: .asciz "Time inteval!\n"

- IN_ADDRESS_HEXA_KEYBOARD (0xFFFF0012): Address for the hexadecimal keyboard interrupt.

- TIMER_NOW (0xFFFF0018): Address of the timer's current value.

- TIMER_CMP (0xFFFF0020): Address to set the timer comparison value.

- MASK_CAUSE_TIMER (4): Mask to identify timer interrupt cause.

- MASK_CAUSE_KEYPAD (8): Mask to identify keypad interrupt cause.

Stores two message strings for interrupts:

- msg_keypad: "Someone has pressed a key!\n"

- msg_timer: "Time interval!\n"

Main Procedure

- Setup for Interrupt Handling:

main:

# Load the interrupt service routine address to the UTVEC register

la t0, handler

csrrs zero, 5, t0

- la t0, handler: Load the address of the handler (interrupt service routine) into register t0.

- csrrs zero, 5, t0: Store the address in the utvec CSR (control and status register), which sets the interrupt handler.

- Enable External and Timer Interrupts:

# Set the UEIE (User External Interrupt Enable) bit in UIE register

li t1, 0x100

csrrs zero, 4, t1 # uie - ueie bit (bit 8) - external interrupt

csrrsi zero, 4, 0x10 # uie - utie bit (bit 4) - timer interrupt

- li t1, 0x100: Load 0x100 (bit 8) to enable external interrupts.

- csrrs zero, 4, t1: Set bit 8 in UIE register (User Interrupt Enable).

- csrrsi zero, 4, 0x10: Set bit 4 in UIE register to enable timer interrupts.

- Enable Global Interrupts:

# Set the UIE (User Interrupt Enable) bit in USTATUS register

csrrsi zero, 0, 0x1 # ustatus - enable uie - global interrupt

- csrrsi zero, 0, 0x1: Enable global interrupts by setting the UIE bit in the ustatus register.

- Enable Keypad and Timer Interrupts:

# Enable the interrupt of keypad of Digital Lab Sim

li t1, IN_ADDRESS_HEXA_KEYBOARD

li t2, 0x80 # bit 7 of = 1 to enable interrupt

sb t2, 0(t1)

- li t1, IN_ADDRESS_HEXA_KEYBOARD: Load the address of the keypad interrupt control register.

- li t2, 0x80: Set bit 7 to enable keypad interrupts.

- sb t2, 0(t1): Store the value 0x80 in the memory location, enabling the keypad interrupt.

# Enable the timer interrupt

li t1, TIMER_CMP

li t2, 1000

sw t2, 0(t1)

- li t1, TIMER_CMP: Load the address of the timer comparison register.

- li t2, 1000: Set the timer comparison value to 1000.

- sw t2, 0(t1): Store the value 1000 to start the timer interrupt.

- Main Program loop:

loop:

nop

li a7, 32

li a0, 10

ecall

nop

j loop

end_main:

- This loop keeps the program running indefinitely (nop instructions do nothing).

- li a7, 32 and li a0, 10 prepare for an environment call (ECALL) to delay for 10ms.

- ecall invokes the delay system call.

Interrupt Service Routine

- Save the Context:

handler:

# Saves the context

addi sp, sp, -16

sw a0, 0(sp)

sw a1, 4(sp)

sw a2, 8(sp)

sw a7, 12(sp)

- Save the registers a0, a1, a2, and a7 onto the stack to preserve the context during the interrupt.

- Handle Timer and Keypad Interrupts:

# Handles the interrupt

csrr a1, ucause

li a2, 0x7FFFFFFF

and a1, a1, a2 # Clear interrupt bit to get the value

- csrr a1, ucause: Load the cause of the interrupt into a1.

- and a1, a1, a2: Mask out the interrupt bit to isolate the interrupt cause.

- Timer Interrupt Handling:

li a2, MASK_CAUSE_TIMER

beq a1, a2, timer_isr

- Check if the interrupt cause matches the timer interrupt mask (4).

- If yes, branch to timer_isr.

timer_isr:

li a7, 4

la a0, msg_timer

ecall

# Set cmp to time + 1000

li a0, TIMER_NOW

lw a1, 0(a0)

addi a1, a1, 1000

li a0, TIMER_CMP

sw a1, 0(a0)

j end_process

- Prints the message "Time interval!" to indicate that a timer interrupt occurred.

- Schedules the next timer interrupt after a delay of 1000 time units by updating the TIMER_CMP register.

- Returns to the main program after completing the interrupt service routine.

- Keypad Interrupt Handling:

li a2, MASK_CAUSE_KEYPAD

beq a1, a2, keypad_isr

- Check if the interrupt cause matches the keypad interrupt mask (8).

- If yes, branch to keypad_isr.

keypad_isr:

li a7, 4

la a0, msg_keypad

ecall

j end_process

- Print the keypad message.

- Restore Context and Return:

end_process:

# Restores the context

lw a7, 12(sp)

lw a2, 8(sp)

lw a1, 4(sp)

lw a0, 0(sp)

addi sp, sp, 16

uret

- Restore saved registers from the stack.

- uret: Return from the interrupt.

Result: