Lab Report
Lab 7: Subroutine Call and Passing Parameters Using the Stack

| Student ID | 20235890 |
| --- | --- |
| Student Name | Nguyễn Đức Anh |

Assignment 1:

Code:

# Lab 7, Assignment 1

.text

main:

input:

li a7, 5    # input a integer

ecall       # system call

jal abs     # jump and link to abs procedure

li a7, 10   # terminate

ecall       # system call

end_main:

abs:

sub s0, zero, a0    # put -a0 in s0; in case a0 < 0

blt a0, zero, done  # if a0<0 then done

add s0, a0, zero    # else put a0 in s0

done:

jr ra   # jump to return address

Explaination: Using system call to input a value to a0 and do abs procedure and save the value on s0.

Result:

Input:

Output:

Assignment 2:

Code:

# Lab 7, Assignment 2

.text

main:

li a0, 4    # load test input

li a1, 12

li a2, 9

jal max     # call max procedure

li a7, 10   # terminate

ecall

end_main:

max:

add s0, a0, zero    # copy a0 in s0; largest so far

sub t0, a1, s0      # compute a1 - s0

blt t0, zero, okay  # if a1 - v0 < 0 then no change

add s0, a1, zero    # else a1 is largest thus far

okay:

sub t0, a2, s0      # compute a2 - v0

blt t0, zero, done  # if a2 - v0 <0 then no change

add s0, a2, zero    # else a2 is largest overall

done:

jr ra   # return to calling program

Explaination: Finding the maximum value on three parameters a0, a1, a2 and store on s0 by max procedure.

Result:

Input:

Output:

Assignment 3:

Code:

# Lab 7, Assignment 3

.text

li s0, 0x00000005

li s1, 0xfffffffe

push:

addi sp, sp, -12 # adjust the stack pointer

sw s0, 8(sp) # push s0 to stack

sw s1, 4(sp) # push s1 to stack

work:

sum:

add s2, s0, s1  # s2 = s0 + s1

push_s2:

sw s2, 0(sp)    # push s2 to stack

pop:

lw s0, 0(sp) # pop from stack to s0

lw s1, 4(sp) # pop from stack to s1

lw s2, 8(sp) # pop from stack to s2

addi sp, sp, 12 # adjust the stack pointer

Explaination: Using stack to push s0, s1, the sum s2 = s0 + s1 and pop it by swapping the value s0 = s2, s1 = s1 and s2 = s0.

Result:

Input:

Sum:

Output:

Assignment 4:

Code:

# Lab 7, Assignment 4

.data

message: .asciz "Ket qua tinh giai thua la: "

.text

main:

jal WARP

print:

add a1, s0, zero # a0 = result from N!

li a7, 56

la a0, message

ecall

quit:

li a7, 10 # terminate

ecall

end_main:

# ----------------------------------------------------------------------

# Procedure WARP: assign value and call FACT

# ----------------------------------------------------------------------

WARP:

addi sp, sp, -4 # adjust stack pointer

sw ra, 0(sp) # save return address

li a0, 3 # load test input N

jal FACT # call fact procedure

lw ra, 0(sp) # restore return address

addi sp, sp, 4 # return stack pointer

jr ra

wrap_end:

# ----------------------------------------------------------------------

# Procedure FACT: compute N!

# param[in] a0 integer N

# return s0 the largest value

# ----------------------------------------------------------------------

FACT:

addi sp, sp, -8 # allocate space for ra, a0 in stack

sw ra, 4(sp) # save ra register

sw a0, 0(sp) # save a0 register

li t0, 2

bge a0, t0, recursive

li s0, 1 # return the result N!=1

j done

recursive:

addi a0, a0, -1 # adjust input argument

jal FACT # recursive call

lw s1, 0(sp) # load a0

mul s0, s0, s1

done:

lw ra, 4(sp) # restore ra register

lw a0, 0(sp) # restore a0 register

addi sp, sp, 8 # restore stack pointer

jr ra # jump to caller

fact_end:

Explaination: The program implement how to calculate the factorial of n by recursive algorithms and stack.

- The Warp procedure initials input for calculating factorial and the return address ra on the stack, after the Fact return, Warp restores the ra register and stack pointer before jumping back to main.

- The Fact procedure calculates the factorial recursively. It checks if N is greater than or equal to 2. If N is 1, it returns 1. If N is 3 (or any number >= 2), it decrements N (a0) by 1 and calls itself recursively. On each recursive return, Fact multiplies the current result (s0) by N, saved in s1. When the recursion completes, it restores a0, ra, and the stack pointer before returning to Warp.

Result:

Output:

Assignment 5:

Code:

.data

message_largest: .asciz "Largest: "

message_smallest: .asciz "\nSmallest: "

comma: .asciz ", "

.text

main:

# Call Max_Min subroutine

jal Max_Min

print:

# Print largest value and its position

la a0, message_largest    # Load message for largest

li a7, 4                  # System call for print string

ecall

addi a0, s0, 0            # Move largest value to a0 for print

li a7, 1                  # System call for print integer

ecall

li a7, 4                  # Print comma and space

la a0, comma

ecall

addi a0, s1, 0            # Move largest position to a0 for print

li a7, 1                  # System call for print integer

ecall

# Print smallest value and its position

la a0, message_smallest   # Load message for smallest

li a7, 4                  # System call for print string

ecall

addi a0, s2, 0            # Move smallest value to a0 for print

li a7, 1                  # System call for print integer

ecall

li a7, 4                  # Print comma and space

la a0, comma

ecall

addi a0, s3, 0            # Move smallest position to a0 for print

li a7, 1                  # System call for print integer

ecall

li a7, 10                 # System call for program exit

ecall

Max_Min:

# Initial example values in a0 to a7

li a0, 2

li a1, -1

li a2, 4

li a3, 9

li a4, 5

li a5, 3

li a6, 21

li a7, 7

# Store initial values

addi sp, sp, -16       # Adjust stack for storing registers

sw ra, 12(sp)          # Store return address

sw t0, 8(sp)           # Store temporary registers

sw t1, 4(sp)

sw t2, 0(sp)

# Initialize largest and smallest values with a0 and positions with 0

addi s0, a0, 0         # s0 = largest value

addi s2, a0, 0         # s2 = smallest value

li s1, 0               # s1 = position of largest value

li s3, 0               # s3 = position of smallest value

# Begin comparisons with registers a0 through a7

li t0, 0               # Position counter for current register, start from 0

# Manually check each register from a0 to a7

addi t1, a0, 0         # Load a0

jal compare_and_update

addi t0, t0, 1         # Increment position

addi t1, a1, 0         # Load a1

jal compare_and_update

addi t0, t0, 1         # Continue incrementing position

addi t1, a2, 0         # Load a2

jal compare_and_update

addi t0, t0, 1

addi t1, a3, 0         # Load a3

jal compare_and_update

addi t0, t0, 1

addi t1, a4, 0         # Load a4

jal compare_and_update

addi t0, t0, 1

addi t1, a5, 0         # Load a5

jal compare_and_update

addi t0, t0, 1

addi t1, a6, 0         # Load a6

jal compare_and_update

addi t0, t0, 1

addi t1, a7, 0         # Load a7

jal compare_and_update

end_loop:

# Restore the stack and return

lw ra, 12(sp)          # Restore return address

lw t0, 8(sp)           # Restore temporary registers

lw t1, 4(sp)

lw t2, 0(sp)

addi sp, sp, 16        # Restore stack pointer

jr ra                  # Return to caller

compare_and_update:

bgt t1, s0, update_largest  # If current value > largest, update largest

blt t1, s2, update_smallest # If current value < smallest, update smallest

jr ra                       # Return to caller

update_largest:

addi s0, t1, 0              # Update largest value

addi s1, t0, 0              # Update largest position

jr ra                       # Return to caller

update_smallest:

addi s2, t1, 0              # Update smallest value

addi s3, t0, 0              # Update smallest position

jr ra                       # Return to caller

Explaination:

- Procedure Max_Min: Find the largest and smallest values and their positions from registers a0 to a7.

Returns:

  - s0 - largest value

  - s1 - position of largest value (0 to 7)

  - s2 - smallest value

  - s3 - position of smallest value (0 to 7)

- Procedure compare_and_update: Compares current value (t1) with current largest (s0) and smallest (s2), updates values and positions if necessary.

Result: